Documentation of CR01e:

# Program of 2-D Discrete Element Method with Circular Elements

## Takashi Matsushima

Institute of Engineering Mechanics and Systems, University of Tsukuba

1-1-1 Tennodai, Tsukuba, Ibaraki, 305-8573 JAPAN

tmatsu@kz.tsukuba.ac.jp

1. CR01e-1 PACKAGE

CR01e package contains the following programs:

[1]Pre-processors

*Egen11exp, Egen11r  …for the generation of the grains and the piling them up

*Arrange		    …for changing the shape of the packed specimen, setting the fixed grains, etc.

[2]Main DEM program

*CR01e

[3]Post-processors

*DEMView42e	…for drawing the calculation result, making the GIF animation, etc.

2. PRE-PROCESSING TOOLS

2-1. Egen11e

Egen11e is a program for random grain generation within a certain rectangular domain. Before executing it, you have to edit “egenctrl.dat” as shown in Figure 1.

You can generate the grains according to the two distributions; a normal distribution (kdist=0) and a log-normal distribution (kdist=1). The latter distribution is more suitable for the well-graded assembly than the former one as shown in Figures 2 to 5. It is noted that the percent passing by number of grains is different from that by weight which is normally used in sands. Since the input parameters SIZEM and SIZEV in egen11exp controls the mean grain size and the standard deviation of the assembly, the value of so-called D50 varies even through SIZEM keeps unchanged. Therefore we have decide these parameters by trial and errors in order to obtain the assembly with a certain D50.

0	2

500       10000

1.0        0.3       0.0        2.0

-30.0     -30.0     30.0     30.0

0

for CR01e (normal distribution in grain size)

======================================================================

kdist, jtype

kdist=0...normal distribution for grain size distribution

=1...log normal distribution for grain size distrib.

jtype=0...Out.dat for the old CR and EL grain type

=1...for regulrar polygonal elements,      =2...Simple CR01 type

NOP, LOOP	...Number of grains,  Maximum number of iterations

SIZEM, SIZEV, sizemn, sizemx

...The average and the standard deviation of grain size, and the minimum and the maximum values

X1,Y1,X2,Y2	Grain generation domain (X1,Y1)-(X2, Y2)

NOW		Number of the Additional lines to control the grain generation domain

DO 13 I13=1,NOW

READ(2,*) JP(I13),AW(I13),BW(I13)	Each line; y>(or<) AW*x+BW

13 CONTINUE

if(jtype.eq.0) READ(2,*) AXISM,AXISV,axismn,AXISMX	...for the distribution of aspect ratio

if(jtype.eq.1) read(2,*) dndgm,dndgv,ndgmn,ndgmx

...for the distribution of the shape of the regular polygonal grains

Figure 1   Example of “egenctrl.”

(a) narrow range (sizem=1.0, sizev=0.3)			(b) wide range (sizem=1.0, sizev=0.6)

Figure 2   Results of Egen11e (1):  normal distributions (from “newin.dat”)

(a) Percent by the number of grains			(b) Percent by the weight of grains

Figure 3   Results of Egen11e (2):  normal distribution curves (from “outd.dat”)

(a) narrow range (sizem=1.0, sizev=0.6)			(b) wide range (sizem=1.0, sizev=1.2)

Figure 4  log-normal distributions (from “newin.dat”)

(a) Percent by the number of grains			(b) Percent by the weight of grains

Figure 5  Example of log-normal distribution curves (from “outd.dat”)

2-2. Egen11r

Egen11r is a program to generate the grains according to a certain experimentally-derived grain-distribution. Based on this distribution by weight, egen11r computes at first the minimum number of grains with which the given distribution can be appropriately reproduced. You can magnify this number by the parameter “impow” in the ctrlr.dat. In the example shown in Figure 6, we divide the grain-size range into 7 and give the percent passing by weight for each division, which is indicated as “model curve” in Figure 7(a). In each division, the program generate the corresponding number of grains in a uniform way, and output the grain information to “newin.dat” as shown in Figure 7(b). This newin.dat is used in the main program CR01e.

(a) Grain size distribution curves		(b) Result of random grain generation by egen11r

Figure 7  An example of egen11r

7

0.075		0.0

0.106		0.087

0.25		0.283

0.425		0.3913

0.85		0.5435

2		0.7609

4.75		1.0

2	2000	    2

0.0        0.0        10.0       10.0

0

0

======================================================================

kdiv			Number of grain diameter ranges

ddiv(i)	pbw(i)		Grain diameter range,  Percent passing by weight

jtype, LOOP, impow	...	jtype…Grain type, (old CR & EL..0, RP…1,  New CR01e…2

LOOP…Maximum iteration number

impow…magnifying power for the minimum number of grains

(minimum number of grains is computed in the program.)

X1,Y1,X2,Y2	Grain generation domain (X1,Y1)-(X2, Y2)

NOW		Number of the Additional lines to control the grain generation domain

DO 13 I13=1,NOW

READ(2,*) JP(I13),AW(I13),BW(I13)	Each line; y>(or<) AW*x+BW

13 CONTINUE

if(jtype.eq.0) READ(2,*) AXISM,AXISV,axismn,AXISMX	...for the distribution of aspect ratio

if(jtype.eq.1) read(2,*) dndgm,dndgv,ndgmn,ndgmx

...for the distribution of the shape of the regular polygonal grains

Figure 6   Example of “ctrlr.dat” for egen11r

2-3. ARRANGE

Program ARRANGE is used to process the input.dat file for CR01e. You can move the model in x- and y- direction by task(2), and rotate the model by task(1). Tasks(4) and (6) are used to cut out a certain grains from an original data, which is useful to make a model with non-rectangular boundaries such as embankment models, etc.

Tasks(7)(8)(9) and (10) are used to set grains attaching and fixing in walls. The usage is described later in details.

Figure 8  Possible jobs in Arrange.for

3.  CR01e: THE MAIN DEM PROGRAM

3-1. Input files

CR01e requires the following four input files:

(1) input.dat	…has the information on grains and walls

(2) bcond.dat	…controls the conditions of calculation

(3) cldsp.dat	…uses in the case of the successive calculation

(4) perip.dat	…requires when you use a periodic boundary

3-1-1. input.dat

Figure 9 shows an example of input.dat, which is generated by egen11exp.dat as newin.dat. In this example, density of grains and the gravitational acceleration are set to 0.025 and 9800 (see line [8]), respectively, which means the adopted unit system is (mm, g, s). The spring constants and damping coefficients at the contact points are independent from the confining pressure in CR01e (see lines[10] and [11]). If you want to introduce a spring according to Hertz theory, you have to modify the program.

In order to determine the spring constants, kn and ks (normal and shear springs), the following approximated relations are available (Meguro, 1991).

0									[1]

500      4               .0000020       0						[2]

1.318337145260771								[3]

-14.017083798688020        5.683487123167843        1.304271654067510		[4]

-14.017083798688020        5.683487123167843        1.304271654067510		[5]

1.315739055788425

7.728928970304011      -28.466819785629450       -1.168402808031642

7.728928970304011      -28.466819785629450       -1.168402808031642

----omitting lines-----

4.670120620755326E-001

2.353263831022034       24.840724036813430   4.236391261117147E-001

2.353263831022034       24.840724036813430   4.236391261117147E-001

-.3000000E+02      .3000000E+02     -.3000000E+02     -.3000000E+02			[6]

-.3000000E+02     -.3000000E+02      .3000000E+02     -.3000000E+02

.3000000E+02     -.3000000E+02      .3000000E+02      .3000000E+02

.3000000E+02      .3000000E+02     -.3000000E+02      .3000000E+02

1										[7]

.2500000E-01      .9800000E+04							[8]

.5000000E+00      .5000000E+00      .0000000E+00					[9]

.0000000E+00      .9600000E+09      .2400000E+09					[10]

2000.000000000000000     1000.000000000000000					[11]

======================================================================

[1]	Identification number for grain type (put 0 in CR01e)

[2]	number of grains, NOP, number of walls, NOW, time increment, TSTEP

number of grains attaching to the first displacement-controlled wall, NOFP

[3]	radius of the first grain

[4]	x-, y- coordinates of the center of the first grain, and the inclination angle (rad.) at the time (t=-dt)

[5]	x-, y- coordinates of the center of the first grain, and the inclination angle (rad.) at the time (t=0)

Lines [3][4] and [5] are repeated for NOP times

[6]	position of the first wall element (xw1,yw1)-(xw2,yw2)

line [6] is repeated for NOW times

[7]	number of calculation steps, NLOOP

[8]	density of grains, rho, gravitational acceleration, g

[9]	friction coefficient between grains, friction coefficient between a grain and a wall

[10]	damping coefficient of pore material (use 0 in usual case), spring constants for normal and shear springs, kn, and ks

[11]	damping coefficients of the normal and shear dashpots

Figure 9   Example of “input.dat.”

kn=πρVp24,	ks=πρVs24

Figure 10  Contact model in DEM

akn

cn

cs

aks

where ρ is a density of grains, Vp and Vs are the velocities of compressive and shear waves, respectively. Since this relation is derived based on the wave transmission in a 1-D grain column, you have to check those values by a wave-transmission simulation after packing your assembly.

For the damping coefficients, cn and cs, (see line [11]) the following inequality should be satisfied to avoid the over-damping phenomenon:

cn≤2rminπ ρ kn	cs≤2rminπ ρ ks

where rmin is the radius of the smallest grain in the model.

The determination of the time increment, Δt (see line [2]) is quite important for the stability of the calculation. Meguro derived the following stability condition based on that in Finite Difference Method (Meguro 1991):

Δt≤2rminVp

On the other hand, Matsushima suggests the following inequality based on the minimum requirement of nc; the calculation-step number during a simple contact (Matsushima 1997):

Δt≤πnc2rminVp

where nc should be bigger that 10 for the better quality of the calculation.

For the friction coefficient between grains, μ, many experimental researches shows that it is around 20 (deg.) to 26 (deg.). (Rowe, 1962, Mogami, 1969)

3-1-2. bcond.dat

bcond.dat is a input file to control the calculation conditions. For boundary conditions, CR01e contains the following three special boundaries:

(1) Pressure-Controlled (P-C) wall		for the constant pressure condition

(2) Displacement-Controlled (D-C)wall		for the strain-controlled condition

(3) Periodic boundary			for infinite slope, simple shear test, etc.

Both P-C and D-C walls can stick some grains to model their surface roughness. Note that the grains attaching to the P-C walls are defined in bcond.dat (see line [2]’’), while the grains attaching to the first D-C wall is defined in input.dat (see line [2] in input.dat). NOFP is the number of attached grains, and these grains’ information is shown as the last NOFP grains of all the grains.

See Figures 11 and 12 for simpler example.

0     0      5000      500   0.250D+01   1						[1]

0           0								[2]

0									[3]

0									[4]

======================================================================

(2-1)general parameters on computation

[1]	jcld,iloop,npull,ncstep,cjarea,ldwall

jcld=0:	non-successive calculation, 	jcld=1: successive calculation

iloop	number of calculation steps at the last calculation (valid in the successive calculation)

npull	In every npull steps, the program outputs the data to ‘output.dat’

ncstep	in every ncstep steps, the contact-judgment area is re-constructed.

cjarea	The radius of contact-judgment area

ldwall	the wall number to monitor the load-displacement data

(2-2)Setting of Pressure-controlled walls

[2]	nomw,nopps	number of pressure-controled walls, number of attached grains in each p-c wall

[2]’		domx(i70),domy(i70),rowow(i70)

(domx,domy) shows the orientation of the pressure vector

rowow is a magnitude of the pressure ( Actually rowow*g shows a pressure.)

line [2]’ is repeated nomw times

[2]’’	nwpsn(i81),nppsn(i81)		number of walls to which the grain is attached, grain number

line [2]’’ is repeated nopps times

(2-3)Setting of Displacement-controlled walls

[3]	nomw2	number of d-c walls

[3]’		wdspx,wdspy	velocities of the first d-c wall in x- and y- directions, respectively

line [3]’ is repeated nomw2 times

(2-4)Setting of periodic boundary

[4]	nper	nper=0(no periodic boundary),1(x-direction periodic boundary)

[4]’		xper1,xper2,widpr,rdpr		periodic region is [xper1-xper2] in x- coordinates

widpr is the contact judgment area. widpr > (xper2-xper1)

rdpr is a speed to reduce the periodic region (usually set to 0)

Figure 11   Example of “bcond.dat.”

3-1-3. Successive calculation

If you want to conduct the successive calculation of the preceding one, you can use ‘newin.dat’, ‘newbcn.dat’, and ‘newcld.dat’ as ‘input.dat’, ‘bcond.dat’, and ‘cldsp.dat’, respectively. ‘cldsp.dat’ contains the information of the cumulative displacement of each acting shear spring. The batch file ‘continue.bat’ is usuful for this work.

perip.dat is re-constructed automatically at the end of the calculation.

Notes:

(a) input.dat

0

5      4               .0000020       1

1.0			(grain No.1)

5.0	5.0	0.0

5.0	5.0	0.0

1.0			(grain No.2)

22.0	8.0	0.0

22.0	8.0	0.0

2.0			(grain No.3)

2.0	7.0	0.0

2.0	7.0	0.0

3.0			(grain No.4)

10.0	5.0	0.0

10.0	5.0	0.0

4.0			(grain No.5: the last one)

18.0	4.0	0.0

18.0	4.0	0.0

0.0	10.0	0.0	0.0	(Wall No.1)

0.0	0.0	40.0	0.0	(Wall No.2)

25.0	0.0	25.0	10.0	(Wall No.3)

25.0	10.0	-10.0	10.0	(Wall No.4)

xwa	ywa	xwb	ywb

100000

.2500000E-01      .9800000E+04

.5000000E+00      .5000000E+00      .0000000E+00

.0000000E+00      .9600000E+09      .2400000E+09

2000.000000000000000     1000.000000000000000

(b) bcond.dat

0     0      500      500   0.250D+01   1

1           1			(One P-C wall (No.1) with one attached grain.)

1.0d0	0.0d0	0.0025d0		(P-C wall acts only to the x-direction.)

1	3			(The grain attaching to wall No.1 is the 3rd grain.)

1				(One D-C wall (No.2))

-5.0d1	0.0d0			(D-C wall moves in –x direction at the speed, 50.0)

0				(No periodic boundary)

Figure 12   Example of P-C and D-C walls

(xwa,ywa)

(xwb,ywb)

(a) right setting of wall element

(xwb,ywb)

(xwa,ywa)

(b) wrong setting of wall element

Figure 13 Setting of wall element

Static wall (No.3 & 4)

grain No.3

P-C wall (No.1)

D-C wall (No.2)

grain No.5 (the last one)

(i) Any wall element is valid only from one side (Figure 13). If a grain comes from the wrong side, it passes through the wall. [(xwa(1),ywa(1))=(0,10), (xwb(1),ywb(1))=(0,0) in Figure 12.]

(ii) In input.dat, P-C walls come first, D-C walls come second, and the rest walls are static walls. In Figure 12, we have one P-C wall, and one D-C wall. So the No.1 wall corresponds to the P-C wall, and the No.2 wall corresponds to the D-C wall. The rest No.3 and 4 walls are static walls.

(iii) If the number of grains attaching to the D-C wall is 2, the last two grains in input.dat (grain No.4 and No.5 in Figure 12) moves with the D-C wall.

3-2. Output files

CR01e generates the following output files:

(1) newin.dat, newbcn.dat, newcld.dat…	for the successive calculation (see 3-1-3)

(2) output.dat…	Positions of grains in every npull steps, which is used to view the result with DEMViewer.

(3) wllout.dat…	Positions of walls in every npull steps, which is also used in DEMViewer. When you use only static walls, you don’t need this file.

(4) cont.dat, cont2.dat…	Information on the contact forces acting on each contact. DEMViewer requires these files if you want to view the information of contact forces.

(5) lddsp.dat…	a load-displacement data of the D-C wall.

(6) energy.dat…	Energy balance throughout the calculation

(7) check.dat…	Error messages

(8) rfchck.dat, cnfprs.dat…	data files used in simple shear test.

3-2-1. Output.dat

Contents of Output.dat is almost identical to those in input.dat, but the arrangement is different. Information of all grains’ position is output here in every NPULL step. See Figure 14 in details.

3-2-2. Wllout.dat

Information on wall position. See Figure 15 in details.

3-2-3. Cont.dat & Cont2.dat

Cont2.dat has the data of the number of contact points between grains and also between a grain and a wall. In this data, wall numbers are added by the number of grains (NOP). Cont.dat has the detailed data for each contact point, its location (cpx, cpy), the angle of the contact plane, and the normal and shear forces acting on the point.

3-2-4. Energy.dat

Energy.dat provides the information of energy conservation throughout the calculation. It contains all the energies in the system; (1)potential energy by gravity, (2)kinetic energy (consisting of x-, y- and theta components), (3)strain energy of contact springs (normal and shear components), (4)energy dissipation by contact dashpods (normal and shear), (5)energy dissipation by friction (at the contact slider), (6)energy dissipation concerning air damping (see input.dat, line[10] in Figure 9), (7)energies concerning P-C walls (kinetic energy, dissipation of damping, and the work done by Pressure), and (8)the work done by D-C walls. In addition, the error of total energy conservation is output, which is a good measure to check if the calculation is conducted properly.

Note: In old fortran system, the length of a line in an output data file cannot exceed 72 characters. If you use this system, you have to modify the writing format in the program.

0									[1]

500     4   0.200D-05   0.000D+00   0.000D+00   0.000D+00   0.000D+00			[2]

100000									[3]

0.2500000D-01    0.9800000D+04							[4]

0.5000000D+00    0.5000000D+00    0.0000000D+00					[5]

0.0000000D+00    0.9600000D+09    0.2400000D+09					[6]

0.2000000D+04    0.1000000D+04							[7]

1 -.3000D+02 0.3000D+02 -.3000D+02 -.3000D+02					[8]

2 -.3000D+02 -.3000D+02 0.3000D+02 -.3000D+02

3 0.3000D+02 -.3000D+02 0.3000D+02 0.3000D+02

4 0.3000D+02 0.3000D+02 -.3000D+02 0.3000D+02

1   1.31833714526077     							[9]

2   1.31573905578842

4   1.27214080248590

499  0.475635991522022

500  0.467012062075533

1				[10]

1  -0.14017083799D+02   0.56834871232D+01   0.13042716541D+01				[11]

2   0.77289289703D+01  -0.28466819786D+02  -0.11684028080D+01

499   0.10412291732D+02   0.29386014187D+02  -0.13665844321D+01

500   0.23532638310D+01   0.24840724037D+02   0.42363912611D+00

2				[12]

1  -0.14017083799D+02   0.56834871232D+01   0.13042716541D+01

2   0.77289289703D+01  -0.28466819786D+02  -0.11684028080D+01

=======================================================================================

[1]	Identification number for CR01e

[2]	number of grains, NOP, number of walls, NOW, time increment, parameters for periodic boundary (xper1, xper2, widpr, rdpr)

[3]	number of calculation steps

[4]	density of grains	gravitational acceleration

[5]	friction coefficient between grains, friction coefficient between a grain and a wall

[6]	damping coefficient of pore material (use 0 in usual case), spring constants for normal and shear springs, kn, and ks

[7]	damping coefficients of the normal and shear dashpots

*Lines [3] –[6] is identical to lines[7]-[11] in “input.dat”. see Figure 9.

[8]	wall number, the information of the both edges of the wall (xwa,ywa)-(xwb,ywb)

Line [8] is repeated NOW times.

[9]	grain number, radius of the grain.

Line [9] is repeated NOP times.

[10]	number of calculation step (the initial two steps are output first, and the calculation results are output.)

[11]	grain number, the coordinates of the center of the grain (xc, yc), and the inclination angle theta (rad.).

Line [11] is repeated NOP times

[12]-	Next calculation step

Figure 14   Example of “output.dat.”

1									[1]

1 -.3000000000000D+02 -.300000D+02 0.300000D+02 -.3000000000000D+02			[2]

2 -.3000000000000D+02 0.300000D+02 -.300000D+02 -.3000000000000D+02

3 0.3000000000000D+02 0.300000D+02 -.300000D+02 0.3000000000000D+02

4 0.3000000000000D+02 -.300000D+02 0.300000D+02 0.3000000000000D+02

5000									[3]

1 -.3000000000000D+02 -.300000D+02 0.300000D+02 -.3000000000000D+02

2 -.3000000000000D+02 0.300000D+02 -.300000D+02 -.3000000000000D+02

=======================================================================================

[1]	calculation step number.

[2]	wall number, the position of the both edges of the wall (xwa,ywa)-(xwb,ywb)

Line [2] is repeated NOW times.

[3]	Next calculation steps. (Data is output in every NPULL step. Here NPULL=5000.)

Figure 15   Example of “wllout.dat.”

1            0            0							[1]

5000            0            0

10000            2            2							[2]

15000            3            2							[3]

95000          796           48

100000          795           48

=======================================================================================

[1]	calculation step number, number of contact points between grains, number of contact points between a grain and a wall

[2]	Next calculation step

Figure 16   Example of “cont2.dat.”

100 -.7310E+02 0.0000E+00 0.7310E+02 0.0000E+00 0.0000E+00 0.0000E+00 0.0000E+00 0.0000E+00 0.0000E+00 0.0000E+00 0.0000E+00 0.0000E+00 0.0000E+00 0.0000E+00 0.4974E-12

200 -.2939E+03 0.0000E+00 0.2939E+03 0.0000E+00 0.0000E+00 0.0000E+00 0.0000E+00 0.0000E+00 0.0000E+00 0.0000E+00 0.0000E+00 0.0000E+00 0.0000E+00 0.0000E+00 -.1469E-05

300 -.6623E+03 0.0000E+00 0.6623E+03 0.0000E+00 0.0000E+00 0.0000E+00 0.0000E+00 0.0000E+00 0.0000E+00 0.0000E+00 0.0000E+00 0.0000E+00 0.0000E+00 0.0000E+00 -.3920E-05

400 -.1178E+04 0.0000E+00 0.1178E+04 0.0000E+00 0.0000E+00 0.0000E+00 0.0000E+00 0.0000E+00 0.0000E+00 0.0000E+00 0.0000E+00 0.0000E+00 0.0000E+00 0.0000E+00 -.7354E-05

500 -.1842E+04 0.0000E+00 0.1842E+04 0.0000E+00 0.0000E+00 0.0000E+00 0.0000E+00 0.0000E+00 0.0000E+00 0.0000E+00 0.0000E+00 0.0000E+00 0.0000E+00 0.0000E+00 -.1177E-04

=======================================================================================

Structure: Each line contains the calculation step number at first and 15 energy components as follows:

(1)potential energy by gravity, kinetic energy (consisting of (2)x-, (3)y- and (4)theta components), strain energy of contact springs ((5)normal and (6)shear components), energy dissipation by contact dashpods ((7)normal and (8)shear), (9)energy dissipation by friction (at the contact slider), (10)energy dissipation concerning air damping (see input.dat, line[10] in Figure 9), energies concerning P-C walls ((11)kinetic energy, (12)dissipation of damping, and (13)the work done by Pressure), and (14)the work done by D-C walls. In addition, (15)the error of total energy conservation

Figure 18   Example of “energy.dat.”

228   458  0.3720D+01 -0.2858D+02 -0.5190D+00 -0.586081800D+03 -0.157277141D+03		[1]

300   415  0.1623D+02 -0.2639D+02  0.3510D+01 -0.287758836D+06 -0.809317886D+05		[1]’

303   502 -0.3532D+01 -0.3000D+02  0.0000D+00 -0.657937960D+03 -0.230452725D+00		[2]

458   502  0.3341D+01 -0.3000D+02  0.0000D+00 -0.106590897D+04  0.166500987D+03		[2]’

66  -135  0.2417D+02 -0.2828D+02  0.2101D+01 -0.720240974D+05 -0.360120487D+05		[3]

73   182 -0.3740D+01 -0.2607D+02  0.5746D-02 -0.117569566D+06 -0.268847900D+05		[3]’

74   502 -0.2259D+02 -0.3000D+02  0.0000D+00 -0.279453088D+03  0.796028580D+02		[3]”

122  -356 -0.8911D+01 -0.2824D+02 -0.3219D+00 -0.133222892D+06 -0.666114459D+05		[4]

458   502  0.3105D+01 -0.3000D+02  0.0000D+00 -0.501099186D+04 -0.250549593D+04		[4]’

3    52 -0.1794D+02 -0.2225D+02  0.3930D+01 -0.593413329D+06 -0.118688563D+06

4   159 -0.2608D+02 -0.2717D+02  0.1440D+01 -0.101924203D+06 -0.233525809D+05

4   285 -0.2683D+02 -0.2816D+02  0.4211D+00 -0.739689215D+06  0.152426359D+06

4   287 -0.2732D+02 -0.2573D+02  0.3119D+01 -0.224911797D+06 -0.242400462D+05

26  -298 -0.2012D+02 -0.2744D+02  0.1148D+01 -0.124771295D+05 -0.623856477D+04

33   287 -0.2796D+02 -0.2409D+02  0.7691D+00 -0.851741515D+05 -0.278927728D+05

33   349 -0.2895D+02 -0.2203D+02  0.3271D+01 -0.199727827D+06 -0.670310445D+05

=======================================================================================

*Data structure: grain (or wall) number 1, grain (or wall) number 2, x- and y- coordinates of the contact point (cpx,cpy), the angle of contact plane, pthta (rad.), the acting normal force, fn, and the shear force, fs

The data is output when a contact occurs. So you have to compare it with cont2.dat (Figure 16) if you want to know when each contact occurs.

[1]&[1]’	the information of the contacts between grains at calculation step No.=10000 (see Figure 16, line[2])

[2]&[2]’	the information of the contacts between a grain and a wall at calculation step No.=10000 (see Figure 16, line[2])

the number ‘502’ means wall number ‘2’ (NOP=500 is added by the wall number.)

[3],[3]’,[3]” and [4],[4]’	correspond to the information on calculation step No.=15000 (line [3] in Figure 16)

Figure 17   Example of “cont.dat.”

3-3. Structure of CR01e

3-3-1. Outlines

The framework of CR01e is summarized in Figure 19.

MAIN…for controlling the loop for each calculation step

INPUT…	for reading from “input.dat”

CALC…for the calculation of motions of grains and walls

PCONTN…for setting the contact judgment area

PCALCC…Contact judgment and calculation of reaction forces between two grains

WCALCC…Contact judgment and calculation of reaction forces between a grain and a wall

MOVING…for solving the equation of motion for each grain

ENERGY…for writing to “energy.dat”

PRINT…for writing to “output.dat”

NEWIN…for writing to “newin.dat”

Figure 19 Framework of CR01e

Notes:

[1] For saving the calculation time, CR01e sets the contact judgment area. This area is decided for each grain as the circular domain whose center is identical to the grain center and whose radius is CJAREA times as big as the grain radius (CJAREA is set at ‘bcond.dat’). In every NCSTEP step, CR01e searches the grains within this domain and restore the corresponding grain numbers for each grain. After that, contact judgment is made only with these grains in the subsequent calculation steps.

[2] newin.dat and newcld.dat is output in every NPULL steps. Therefore, if the calculation is stopped unexpectedly, you can start the successive calculation with these files from the latest results.

3-3-2. Key variables used in CR01e

| (1)General variables g | gravitational acceleration |
| --- | --- |
| (2)Variables on grains NOP NOFP ra(icon) (xc(icon,1), yc(icon,1)) thta(icon,1) (xc(icon,2), yc(icon,2)) thta(icon,2) am(icon) ai(icon) amyu1 amyu2 row ca | number of grains number of grains attached to the D-C wall or Static wall radius of the grain center of the grain at time (t-⊿t) rotation angle of the grain at (t-⊿t) (rad.) center of the grain at time (t) rotation angle of the grain at time (t) (rad.) mass of the grain inertial moment of the grain friction coefficient between grains friction coefficient between a grain and a wall density of the grain damping coefficient of pore material |
| (3)Variables on walls NOW icon3 (xwa0(icon3), ywa0(icon3)) - (xwb0(icon3), ywb0(icon3)) (xwa(icon3), ywa(icon3)) - (xwb(icon3), ywb(icon3)) | number of walls wall number position of the bothe edges of the wall at time (t-⊿t) position of the bothe edges of the wall at time (t) |
| (4) Variables common in grains and walls akv akh cv ch | normal spring constant (stiffness) at a contact point shear spring constant (stiffness) at a contact point normal damping coefficient at a contact point shear damping coefficient at a contact point |
| (5) Variables on computation tstep nloop loop npull | time increment number of calculation steps present calculation number number of steps for output to “output.dat”, “wllout.dat, “cont.dat”, and “cont2.dat”. |

Figure 21  Contact model in DEM

Figure 20  Basic variables on grains and walls

akn

cn

cs

aks

(xc(icon), yc(icon))

(xc(icon2), yc(icon2))

ra(icon)

ra(icon2)

(xwa(icon3), ywa(icon3))

(xwb(icon3), ywb(icon3))

4. POST-PROCESSING TOOLS

4-1. Batch files for data storage

This package contains the following batch files available in DOS system for the efficient data storage:

(1)change.bat	…makes a new directory (folder) and copy all the necessary files into it. You input the command in dos shell as follows:

change 0001

Then a new directory, data0001 is created and the data files are copied as follows:

input.dat   ->  data0001/in-0001.dat

bcond.dat  ->  data0001/bcd-0001.dat

cldsp.dat  ->  data0001/cld-0001.dat

….

You can use some characters instead of numbers: ex) change ex01

(2)bring.bat	…uses as follows:

bring 0001

Then the four input files necessary to run CR01e are copied from data0001 directory.

(3)bringn.bat	…uses just as “bring.bat”, but the resulting data files for successive calculation are copied from data0001 directory. In other words, you can use this command if you want to conduct the successive calculation of data0001.

(4)continue.bat	…uses just by typing “continue”. Then the successive calculation of the previous one is ready. If you wand to store the previous calculation data, use (1)change command before using this command. Otherwise, a part of the data of the previous calculation is overwritten.

4-2. DEMViewer

4-2-1. Outlines

DEMViewer is a Windows program to draw the results obtained by CR01e. It is written in Visual Basic Version 5 and available in Windows 98 and 2000 on Intel processor. This package contains DEMViewer Version 4.2e. If you click “DEMview43e.exe”, you can get an initial window of DEMViewer as shown in Figure 22.

DEMViewer can see both “input.dat” and “output.dat”; you can check visually the input file if it’s in good arrangement, and see the calculation result. “Open Input.dat” in File tab gives you the image of input.dat, while “Open Output.dat” provides the calculation result (Figure 23).

4-2-2. Drawing Input.dat

After choosing the proper file, you can get the image of “input.dat” and the following operations become possible with the image operation buttons (Figure 24):

(1) Zoom-in and Zoom-out, (2) Vertical and horizontal movement of the view-window

4-2-3. Drawing Output.dat

Figure 22 Initial window of DEMViewer

Figure 23 File tab in DEMViewer

Figure 24 Image operation buttons

When you open the file, you’ll have a caution that “wllout.dat” is also mounted together with “output.dat”. Press just “OK”, and if you choose the data file with the format of “output.dat”, you can get a good image of the initial condition of the calculation. Then you can use the animation buttons (Figure 25) to see the motion of grains and walls successively.

Figure 25 Animation buttons for “output.dat”

(1) “step by step” button is used to see the motion step by step.

Figure 26  Buttons for drawing contact forces

(2) “start” button is used to see the animation.

(3) “stop” button is used to stop the animation.

(4) “Reset” button is used to go back to the first step.

Note that this program doesn’t have step-by-step rewinding.

(5) “speed” bar and “nskip” button is used to get the faster animation.

Figure 26 shows the buttons concerning the contact forces. When you want to draw contact forces or relative rotations between grains, you can use these buttons. “Cont.dat” and “cont2.dat” are mounted when something changes with these buttons. In “draw lines” mode, contact forces are indicated with red lines. The contacts in which the acting normal force, fn, is bigger than the value, (average fn)*(ratio), are shown with thick lines, while the others are shown with thin lines (Figure 27(a)). The threshold parameter, ratio, can be changed by clicking “ratio” button. In “grain select” mode, only the grains transmitting the bigger contact forces (fn>fn_ave*ratio) are drawn; the grain column forming in the assembly can be seen (Figure 27(b)). “Relative rotation button enables to see the contact points with a blue square, in which the relative rotation between the two grains are bigger than the average value (Figure 27(a)).

(a) “draw lines” mode				(b) “grain select” mode

Figure 27	  The Indication of contact forces and relative rotations

“Color ON” button enables you to see the direction that grains rotates. Yellow grains are rotating clockwise, while green grains are rotating anti-clockwise. Grey ones have no rotational component.

You can also use the “Periodic” button when you want to draw the periodic grains.

4-2-4. Other file operations

In “Edit” tab, you can find “copy” command with which the image is copied in the clipboard. Therefore, you can paste the image to any windows application as a form of BMP file.

In “Animation” tab, a command to make a GIF-animation is available. However it is necessary to put the following two free softwares in the same directory as DEMViewer exists:

I_view32.exe

gifanim.exe

The information of these softwares are available in http://www.forest.impress.co.jp/ (in Japanese) or others.

5. CALCULATION EXAMPLES

This section provides examples to conduct a DEM simulation together with the detailed processes.

5-1. Bearing capacity test

5-1-1. Ground model making

In this example, we began with a result by egen11exp shown in Figure 2(a). The obtained “newin.dat” was renamed into “input.dat”, and changed only the number of “nloop” from 1 to 100,000. Next, “bcond.dat” was made as shown in Figure 28.

0     0      5000      500   0.250D+01   1

0           0

0

0

Figure 28   An example of “bcond.dat”

CR01e can run only with these files successfully, and we can obtain the result shown in Figure 27 after 100,000 steps. This calculation data was stored in DATA001 directory by the following command in DOS prompt:

>change 001

and the preparation of the successive calculation was done by

>continue

5-1-2. Footing model

Edited the new “input.dat” generated by 5-1-1., and added the footing model with the combination of wall elements as shown in Figure 29. The adequate position of the footing can be visually checked with DEMViewer. In addition, new “bcond.dat” was also modified as shown in Figure 30. ldwall is set to 3 in order to obtain the load-displacement curve at the bottom of the footing.

5-1-3. Calculation results

Cr01e was executed and the deformation pattern as shown in Figure 31 was obtained. Load-displacement curve (Figure 32) was obtained from “lddsp.dat”, in which (1)horizontal position, (2) vertical position, (3) horizontal reaction force, (4) vertical reaction force were output.

0

500     9   0.200000D-05     0

0.131834D+01

-.13415898769778D+02  -.11134999984854D+02  0.12895277467062D+01

-.13415898741531D+02  -.11134999966083D+02  0.12895277789619D+01

0.131574D+01

0.73902298499454D+01  -.28684294149583D+02  -.10399890792232D+01

(omit the central part)

0.12746729944090D+02  0.10444915417809D+01  0.47045171054010D+01

0.467012D+00

0.27416068407602D+01  -.64288514650785D+00  -.19548967115853D+00

0.27416069071400D+01  -.64288503565589D+00  -.19548970315758D+00

5.0d0	35.0d0	5.0d0	4.0d0

5.0d0	4.0d0	4.0d0	3.0d0

4.0d0	3.0d0	-4.0d0	3.0d0

-4.0d0	3.0d0	-5.0d0	4.0d0

-5.0d0	4.0d0	-5.0d0	35.0d0

-.30000D+02 0.30000000000000000D+02 -.30000D+02 -.30000000000000000D+02

-.30000000000000000D+02 -.30000D+02 0.30000000000000000D+02 -.30000D+02

0.30000000000000000D+02 -.30000D+02 0.30000000000000000D+02 0.30000D+02

0.30000000000000000D+02 0.30000D+02 -.30000000000000000D+02 0.30000D+02

100000

0.2500000D-01    0.9800000D+04

0.50000000000000D+00  0.50000000000000D+00  0.00000000000000D+00

0.00000000000000D+00  0.96000000000000D+09  0.24000000000000D+09

0.2000000D+04    0.1000000D+04

Figure 29   modeling a footing in “input.dat”

1     100000      5000      500   0.250D+01   3

0            0

5

0.0d0	-10.0d0

0.0d0	-10.0d0

0.0d0	-10.0d0

0.0d0	-10.0d0

0.0d0	-10.0d0

0

Figure 30   new “bcond.dat”

(a) initial cofiguration		     (b) after some settlement with contact force net (100,000 steps)

Figure 31  DEM simulation of a bearing capacity test

NOW was changed into 9.

These five lines for a footing model were added.

ldwall was set to 3

Motion of footing model was set (vy=-10.0).

(a) load-displacement curve (original data was plotted.)		(b) load-displacement curve after smoothing

Figure 32 load-displacement curve

### REFERENCES

Konagai, K. and Matsushima, T.: Key Parameters Governing Dynamic Granular Slope Stability, Journal of Dam Engineering, Vol.7, No.1, pp.27-31, 1997.

Konagai, K., Matsushima, T. and Mikami, A.: Deformation Build up within a Granular Assemblage during an Intense Earthquake, Journal of Earthquake Engineering, Vol. 2, No. 3, pp.419-441, 1998.

Matsushima, T. and Konagai, K.: Failure Process of Coarse Particle Assemblage and its Frequency Dependency, Structural Dynamics -EURODYN’93, Moan et at. (eds.), Trondheim, Norway, Vol.1, pp.373-379, 1993.

Matsushima, T., Grain-to-grain Interaction Affecting Dynamic Deformation of Granular Slope, Dissertation, University of Tokyo, 1997. (in Japanese)

Meguro, K., Study on the dynamic failure of structures by Discrete Element Method, Dissertation, University of Tokyo, 1991. (in Japanese)

Mogami, T., Soil Mechanics, Gihodo, 1969. (in Japanese)

Rowe, P. W., The stress-dilatancy relations for static equilibrium of an assembly of particles in contact, Proc. Royal Soc. London, Series A, 269, 117-141, 1962.